# ERLELO Climate Control System

Version 2.4 - Humidity-Primary Control

Installation & Configuration Guide

NON-NEGOTIABLE TEMPERATURE LIMIT

If chamber temperature exceeds target + hysteresis, the system MUST activate cooling regardless of humidity mode.

# 1. Overview

ERLELO is a multi-chamber curing room climate control system with humidity-primary control.

- 1, 2, or 3 chamber configurations

- Absolute humidity (AH) based deadzone control

- Non-negotiable temperature safety limits

- NTC temperature sensor logging

# 2. File Structure

## 2.1 GitHub Configuration Files

| File | Chambers | Variables |
| --- | --- | --- |
| erlelo_config_1ch.json | 1 | 40 |
| erlelo_config_2ch.json | 2 | 72 |
| erlelo_config_3ch.json | 3 | 104 |

# 3. Installation Steps

## Step 1: Upload JSON to GitHub

Choose the appropriate config file and upload to GitHub.

## Step 2: Create Variables

- Open erlelo_create.lua

- Set NUM_CHAMBERS = 1, 2, or 3

- Deploy to Sinum, press Start Install

## Step 3: Build Mapping

- Deploy erlelo_store.lua

- Press Build Mapping button

- COPY the printed MAPPING_VAR_ID!

## Step 4: Configure Controllers

Set these values in each controller file:

local MAPPING_VAR_ID = 42

local MODBUS_SUPPLY_CLIENT = 8

local MODBUS_CHAMBER_CLIENT = 9

## Step 5: Enable Controllers

- Enable erlelo_kulso.lua

- Enable erlelo_kamra1.lua (and kamra2, kamra3 if needed)

# 4. Variable Naming Convention

## Chamber-Specific: _ch1, _ch2, _ch3

kamra_homerseklet_ch1, kamra_para_ch2, signals_ch3

## Global: _glbl

kulso_homerseklet_glbl, variable_name_map_glbl

# 5. Control Modes

- HUMID: AH too high - need dehumidification

- FINE: AH within deadzone - maintain

- DRY: AH too low - need humidification

# 6. Troubleshooting

- MAPPING_VAR_ID not set: Run erlelo_store and copy the ID

- Variable mapping empty: Run erlelo_store after erlelo_create

- Modbus client not found: Check Sinum Modbus settings